ТЕХНИЧЕСКА СПЕЦИФИКАЦИЯ

за

ОБОСОБЕНА ПОЗИЦИЯ № 2 – АБОНАМЕНТНА ПОДДРЪЖКА НА ТЕХНИЧЕСКИ СРЕДСТВА, НА КОИТО СЕ ПОДДЪРЖА НАЦИОНАЛНА БАЗА ДАННИ „НАСЕЛЕНИЕ“

по обществена поръчка с предмет

„АБОНАМЕНТНО ТЕХНИЧЕСКО ПОДДЪРЖАНЕ НА КОМПЮТЪРНА ТЕХНИКА ЗА НУЖДИТЕ НА МРРБ“

СЪСТОЯЩА СЕ ОТ ДВЕ ОБОСОБЕНИ ПОЗИЦИИ

Описание на оборудването

| Позиция | Параметър | Описание | Партиден № на производителя | Бр. | Сериен номер |
| --- | --- | --- | --- | --- | --- |
| 1. |  | Disk Array |  | 1 |  |
|  |  | HP Enterprise Virtual Array EVA 4000 – 2C1D Array | AD554B | 1 | 2S273405SS |
|  | Storage controller | 4U Controller assembly with dual redundant hot-pluggable FC controllers - 4 Gb FC enabled on each FC path and with backward support for 2 Gb FC. 4GB cache per controller pair with hot plug battery backup; Mirrored Write-Back Cache Support; Read-Ahead and Adaptive Read Caching Support Support for true Active-Active operation - single Logical Unit can be load balanced across both the controllers at the same time Support for multiple paths between hosts and storage system for failover and I/O load balancing using industry standard multiple path software | standard |  |  |
|  | HDDs | Drive enclosure M5314 with 1460GB RAW disk capacity achieved with: Add on HP 146GB dual ported FC hard drives 10 000 rpm hot pluggable; Support for 2/4 Gb/s dual-ported 72, 146, 300GB hot-pluggable Enterprise FC hard drives and support for dual-ported Fibre Attached Technology Adapted (FATA) drives in the same disk shelve of Enterprise FC disks. | 293556-B22 | 10 | SG80730B4Z SG80730B6E SG80730B73 SG80730B97 SG80730B9A SG80730B9B SG80730B9C SG80730BA6 SG80730BCJ SG80730BCL |
|  | Management | Add on HP StorageWorks Command View EVA Unlimited TB license and media kit providing Command Line interface CLI and Graphical User Interface GUI for management and monitoring Online RAID Level Migration; Global distributed online hot spare; Online capacity expansion by self-tuning and dynamic load leveling when additional capacity is added; Online volume / LUN expansion Support for local and remote replication for future disaster recovery Add on Management server HP ProlLiant DL380 G5 Base Storage Server in configuration: - 2 x Dual-Core Intel Xeon 5150 (2.67 GHz, 65 Watts, 1333 FSB, 4MB L2 Cache) plus additional HP Xeon 5150 DL380G5 Kit - 1 GB (2 x 512 MB) Standard plus additional HP 2 GB (2 x 1GB) PC2-5300 Fully Buffered DIMM (DDR2-667) with Advanced ECC - 2 x 36GB SAS 10,000 sff (2.5”) - SmartArray P400 Controller with 512MB BBWC (battery backed write cache0 - HP DVD+R/RW 8x slim - Integrated Lights Out 2 Advanced License - Windows Storage Server 2003 Release 2, Standard Edition - Rack mountable 2U | T3726B T3724D AG453A 418323-B21 397411-B21 | 1 1 1 1 1 | N/A 2S273405SR CZC73424HN GB8734RX6W TWW73401UL |
|  | Power and blowers | Dual hot plug power supplies and blowers for each controller and for each drive enclosure | standard |  |  |
|  | Rack | The array will be mounted in 19” Rack 36U Add on HP Universal Rack 10636 G2 with HP side panels (kit of 2 panels), HP power distribution unit 32A PDUs and HP 10K G2 600W Stabilizer Kit | AF012A AF056A 252663-B31 AF062A | 1 1 2 1 | GB472100CX N/A N/A N/A |
| 2. |  | DB Server |  | 2 |  |
|  |  | HP Integrity rx2660 Server with two CPU, each in configuration: | AD245A | 2 | DEH4734268 DEH4734269 |
|  | Processor | EPIC Technology - 2 x Dual-Core Itanium2 64-bit CPUs 1.6 GHz with 18 MB L3 cache each (9MB per core) | AD245A#003 | 1 |  |
|  | RAM | Total 24GB PC2 4200 533MHz Registered DDR2 SDRAM, up to 32 GB with ECC and chip spare capabilities Add on HP 4GB DDR2 (2x2GB) Memory Add on HP 8GB DDR2 (2x4GB) Memory | AD275A AD276A | 2 2 |  |
|  | Storage | Support for up to 8 x Small Form Factor SAS (Serial Attached SCSI) hot pluggable drives | standard |  |  |
|  | HDD | Add on HP 146 GB Small Form Factor SAS 10,000 rpm hot plug HDD configured in RAID 1 | AD333A | 2 |  |
|  | Controller | Add on HP rx2660 embedded SAS RAID controller supports RAID 1 dedicated slot | AD248A#106 | 1 |  |
|  | Expansion Slots | Add on HP rx2660 PCI-x 3 Slot Cage Option - 2 x 64-bit/266MHz PCI-X - 1 x 64-bit/133MHz PCI-X slots | AD246A | 1 |  |
|  | Fibre Channel Controller | Add on HP 4Gb FC single port adapter PCI-X 2.0 266MHz Host Bus adapters | AB378B | 2 |  |
|  | SCSI Controller | Add on HP Dual channel PCI-X Ultra320 SCSI controller with 2 external 68-pin VHDCI | A7173A | 1 |  |
|  | Optical Drive | Add on HP Integrity DVD-ROM Drive | AD142A | 1 |  |
|  | Ports | 2 x 10/100/1000Base T LAN with LAN boot and WOL (wake on LAN) capability and 1 management 10/100Base TX LAN 3 x USB 2.0, 2 x serial RS232 (one general purpose and one for remote console), 1 x VGA supported | standard |  |  |
|  | Form Factor | Rack-mountable 2U Add on HP rx2660 Rackmount Kit | AD253A | 1 |  |
|  | Rack accessories | Add on HP 16A High Voltage Modular PDU | 252663-B24 | 1 |  |
|  | Operating System | Enterprise class UNIX and binary compatible with the existing Application Software Add on HP-UX 11i Version 3 – preinstalled and DVD Add on HP-UX Integrity Foundation Operating Environment w/System up to 2 Proc Per Core License To Use | BA489AA B9430AC | 1 4 |  |
|  | Management and security: | Integrated Integrity iLO2 Management Processor Functionality for remote management and HA monitoring providing: Separate NIC 10/100 base-T for management – supporting telnet and SSL WEB access; Remote power up and power down control; password protection of console ports Event monitoring and notification; Service CPU allowing remote management and diagnostics Add on HP Integrity TPM (Trusted Platform Module) Embedded Security Chip; | standard AB404A | 1 |  |
|  | High Availability | Hot Plug redundant PSUs Add on HP rx2660 Additional Power Supply, Hot Plug redundant FANs, RAID, Dynamic Processor Resilience (DPR) providing online (dynamic) CPU, On line memory page deallocation (if errors occur) without disruption, double in service chip spare | AD254A standard | 1 |  |
|  | Additional Software | Integrated native multi-pathing in the operating system HP-UX 11i v3 compatible with the disk array HP EVA4000 | standard | 1 |  |
| 3. |  | SAN Infrastructure |  | 2 |  |
|  | FC Switches | HP StorageWorks 4/8 SAN Switch each with following configuration | A8000A | 2 | USB716WMW9 USB718WNXK |
|  | Ports | 8 active auto-sensing 4 Gb/s universal FC self-configuring ports (with e-port connectivity), upgradeable to 16 ports; 1 x 10/100 Ethernet (RJ-45) and 1 x serial port for management; hardware enforced zoning | standard |  |  |
|  | Technology | 4 Gb/s FC port | standard |  |  |
|  | Form factor | Rack mount 1U | standard |  |  |
|  | Software | Web enabled management and monitoring; Zoning | standard |  |  |
|  | Host Bus Adapters | single channel 4Gb/s FC adapters | AE311A AD167A | 6 2 | MY50731F7H MY50731FP0 MY50731FP9 MY50731FRL MY50731FS3MY50731FS6 MX3072701E MX30727063 |
|  | Transceivers and cables | Add on HP 4 Gb/s Optical Short Wave Transceivers Small Form Factor Pluggable Add on HP Storage Works LC/LC 2m Cable Add on HP Storage Works LC/LC 5m Cable 8 FC interfaces necessary per switch: | A7446B 221692-B21 221692-B22 | 16 10 6 | N/A N/A N/A |
| 4 |  | Tape drives |  | 2 |  |
|  |  | HP Ultrium 448 SCSI in 1U Rack 1 Drive two drives achieved by adding Add on HP Ultrium 448 Internal Tape Drive installed in the rack mount enclosure | DW028B DW016A | 1 1 | CZC07340Z7 HU10731MHU |
|  | Capacity | Native data capacity of minimum 200GB (uncompressed)/400GB (compressed 2:1) | standard |  |  |
|  | Drive technology | LTO2 drive with native speed of minimum 24 MB/sec and a compressed speed of minimum 64 MB/sec for 2:1 compression | standard |  |  |
|  | Form Factor | Rack mount 1U | standard |  |  |
|  | Interface | SCSI (cable included) Add on HP Cable 12ft External SCSI VHDCI/WIDE 1 for each drve – total 2 cables | 341177-B21 | 2 | N/A |
| 5 |  | UPS |  | 3 |  |
|  |  | MGE Pulsar MX 5000 RT3U and Add on Pulsar MX EXB RT: battery unit | 68 504 | 2 | 1V1H08055 1V1H08056 |
|  |  | MGE Pulsar 7kVA |  | 1 | P160318 |

Задължения на изпълнителя:

* текущо поддържане и ремонт на дефектиралите устройства;

* извършване на месечни профилактични прегледи, съгласно  изискванията  на фирмата производител;

* даване на консултации и указания за правилната експлоатация на  машините;

* сервизната организация се задължава да реагира в срок до 4 часа от уведомлението

Услугите предмет на  договора се извършват в работните дни от 9 до 17:30 часа в ГД „ГРАО“ към МРРБ

Минимални изисквания към участниците:

- Документ от НР за сертифициран сервиз на технически средства (HP Eva 4000/8000, Optical switch, HP Tape и др.);

- Списък на минимум двама експерта, които ще бъдат ангажирани в изпълнението, придружен от трудови договори, като трудовите им договори трябва да са със дата не по-късна от 0.03.2012 във фирмата - изпълнител

Посочените в списъка лица трябва да имат минимум 4 години професионален опит във фирмата изпълнител (или друга такава, оторизирана от HP за сертифициран сервиз)  в областта на поддръжката на високотехнологични сървърни и сторидж решения от типа:

- HP EVA дисков масив (със специализации доказващи опита на специалиста - MASE -  HP Master Accredited System Engineer Storage Solutions и MASE - Master Accredited System Engineer - HP SAN Architect - Data Availability Solutions);

- Високонадеждни сървърни комплекси - HP Integrity Servers (със специализации доказващи опита на специалиста - АSE Accredited System Engineer - HP Integrity Servers  и CSE - Certified System Engineer -Specialty in High Availability - HP-UX 11 v3).

- Опит в поддържането на такава техника – Itanium Servers and HP EVA. С минимум 2 референции за аналогични дейности.

Опитът се доказва и със справка-декларация за участие в сходни проекти, преподписано от участника.

Критерий за оценка: „най-ниска цена“

Срок на изпълнение – 2 години.